# Hardware Setup Guide: Speakers & Lights

This guide explains how to add external speakers and physical lights to your fire station alert system.

## Table of Contents

1. [Speakers Setup](#speakers-setup)

2. [Lights Setup](#lights-setup)

- [Option 1: USB Serial LED Controller (Arduino)](#option-1-usb-serial-led-controller-arduino)

- [Option 2: Philips Hue Smart Lights](#option-2-philips-hue-smart-lights)

- [Option 3: HTTP-based Light Controller](#option-3-http-based-light-controller)

- [Option 4: Commercial Alert Lighting Systems](#option-4-commercial-alert-lighting-systems)

3. [Display Configuration](#display-configuration)

- [Room Displays vs Main Station Displays](#room-displays-vs-main-station-displays)

- [Night Mode for Room Displays](#night-mode-for-room-displays)

4. [Nighttime Mode](#nighttime-mode)

5. [Configuration](#configuration)

6. [Testing](#testing)

─────────────────────────────────────────────────────────

## Speakers Setup

### Option 1: Computer Speakers/Amplifier (Recommended)

Hardware Needed:

- External speakers (powered speakers or passive speakers with amplifier)

- Audio cable (3.5mm jack or USB audio interface)

- **Speaker Wire (Optional):** Mygatti 14AWG Speaker Wire (300 FT, 14/2 Gauge)

- For longer speaker runs or connecting multiple speakers

- Price: ~$45.99 for 300 feet

- Includes polarity markers for correct wiring

Setup Steps:

1. Connect Speakers:

- Connect speakers to your computer's audio output (headphone jack or USB audio interface)

- For louder volume, use powered speakers or connect to an amplifier

- Position speakers strategically throughout the station for maximum coverage

2. Windows Audio Settings:

- Right-click the speaker icon in the system tray

- Select "Open Sound settings"

- Set your external speakers as the default playback device

- Click "Device properties" → "Additional device properties"

- Go to "Levels" tab and set volume to 100%

- Go to "Enhancements" tab and disable any audio enhancements that might reduce volume

3. Browser Audio Settings:

- The system automatically sets audio to maximum volume

- Ensure browser autoplay is enabled (click "Start System" button on dashboard)

- Check browser volume settings (some browsers have separate volume controls)

4. Testing:

- Send a test alert and verify audio plays through external speakers

- Adjust speaker placement and volume as needed

### Option 2: Bluetooth Speakers

Setup Steps:

1. Pair Bluetooth speakers with your Windows computer

2. Set Bluetooth speakers as default audio output in Windows Sound settings

3. Ensure speakers stay connected (some Bluetooth devices disconnect after inactivity)

Note: Bluetooth may introduce slight audio delay. For critical alerts, wired speakers are recommended.

─────────────────────────────────────────────────────────

## Lights Setup

The system supports three types of light controllers. Choose the option that best fits your hardware.

### Option 1: USB Serial LED Controller (Arduino)

Best for: Custom LED setups, DIY projects, cost-effective solutions

Hardware Needed:

- Arduino board (Uno, Nano, or similar)

- RGB LED strip or individual RGB LEDs

- USB cable to connect Arduino to computer

- Appropriate resistors and wiring (varies by LED type)

Arduino Code:

Upload this code to your Arduino:

// Fire Station Alert Light Controller
// Receives commands via Serial (9600 baud)
String command = "";
bool flashing = false;
unsigned long lastFlash = 0;
int flashInterval = 500; // milliseconds
String currentColor = "off";
void setup() {
  Serial.begin(9600);
  // Configure pins for RGB LED (adjust pin numbers for your setup)
  // Example: Common anode RGB LED on pins 9, 10, 11
  pinMode(9, OUTPUT);  // Red
  pinMode(10, OUTPUT); // Green
  pinMode(11, OUTPUT); // Blue
  // Turn off LED initially
  setColor("off");
  Serial.println("Light controller ready");
}
void loop() {
  // Read serial commands
  if (Serial.available() > 0) {
    command = Serial.readStringUntil('\n');
    command.trim();
    processCommand(command);
  }
  // Handle flashing
  if (flashing && (millis() - lastFlash >= flashInterval)) {
    toggleFlash();
    lastFlash = millis();
  }
}
void processCommand(String cmd) {
  if (cmd.startsWith("COLOR:")) {
    String color = cmd.substring(6);
    setColor(color);
    flashing = false;
  } else if (cmd.startsWith("FLASH:")) {
    // Format: FLASH:red:500
    int colon1 = cmd.indexOf(':', 6);
    int colon2 = cmd.indexOf(':', colon1 + 1);
    if (colon1 > 0 && colon2 > 0) {
      String color = cmd.substring(colon1 + 1, colon2);
      flashInterval = cmd.substring(colon2 + 1).toInt();
      setColor(color);
      flashing = true;
      lastFlash = millis();
    }
  } else if (cmd == "STOP" || cmd == "COLOR:off") {
    setColor("off");
    flashing = false;
  }
}
void setColor(String color) {
  currentColor = color;
  // Adjust these values based on your LED type (common anode vs common cathode)
  // This example assumes common cathode RGB LED
  if (color == "red") {
    analogWrite(9, 255);   // Red ON
    analogWrite(10, 0);    // Green OFF
    analogWrite(11, 0);    // Blue OFF
  } else if (color == "blue") {
    analogWrite(9, 0);     // Red OFF
    analogWrite(10, 0);    // Green OFF
    analogWrite(11, 255);  // Blue ON
  } else if (color == "white") {
    analogWrite(9, 255);   // Red ON
    analogWrite(10, 255);  // Green ON
    analogWrite(11, 255);  // Blue ON
  } else {
    // Off
    analogWrite(9, 0);
    analogWrite(10, 0);
    analogWrite(11, 0);
  }
}
void toggleFlash() {
  if (currentColor == "off") {
    // Turn on
    if (currentColor == "red") {
      setColor("red");
    } else if (currentColor == "blue") {
      setColor("blue");
    }
  } else {
    // Turn off
    setColor("off");
  }
}

Configuration:

1. Upload the Arduino code to your board

2. Connect Arduino to computer via USB

3. Note the COM port (Windows Device Manager → Ports)

4. In the browser, when prompted, select the Arduino's COM port

5. Or configure via environment variable:

VITE_LIGHT_TYPE=serial

Note: Web Serial API requires user interaction (clicking a button) to select the port. The system will prompt you on first use.

─────────────────────────────────────────────────────────

### Option 2: Philips Hue Smart Lights

Best for: Easy setup, wireless, no wiring required

Hardware Needed:

- Philips Hue Bridge (required)

- Philips Hue color bulbs or light strips

- Network connection (Hue Bridge must be on same network as computer)

Setup Steps:

1. Set up Hue Bridge:

- Connect Hue Bridge to your network via Ethernet

- Power on the bridge and wait for all LEDs to be solid

- Note the bridge IP address (check router admin panel or use Hue app)

2. Create API Username:

# PowerShell
   Invoke-WebRequest -Uri "http://BRIDGE_IP/api" -Method POST -Body '{"devicetype":"FireStation#AlertSystem"}' -ContentType "application/json"

# Bash/Linux/Mac
   curl -X POST http://BRIDGE_IP/api -d '{"devicetype":"FireStation#AlertSystem"}'

- Press the button on the Hue Bridge

- Within 30 seconds, run this command (replace `BRIDGE_IP` with your bridge IP):

- Copy the `username` from the response (looks like: `abc123def456...`)

3. Find Light IDs:

# PowerShell
   (Invoke-WebRequest -Uri "http://BRIDGE_IP/api/USERNAME/lights" -UseBasicParsing).Content

# Bash/Linux/Mac
   curl http://BRIDGE_IP/api/USERNAME/lights

- Note the light IDs (usually 1, 2, 3, etc.)

4. Configuration:

Create .env` file in `frontend/` directory:

VITE_LIGHT_TYPE=hue
   VITE_HUE_BRIDGE_IP=192.168.1.XXX
   VITE_HUE_USERNAME=your-username-here
   VITE_HUE_LIGHT_IDS=1,2,3

Or configure via browser localStorage (see Configuration section below).

Testing:

- Send a test alert and verify lights flash red (fire) or blue (EMS)

- Lights will automatically turn off when alert is dismissed

Product Setup:

Indoor Lights - Philips Hue Play Light Bar:

- **Product:** Philips Hue Smart Play Light Bar Base Kit (2-Pack, Black)

- **Price:** ~$138.88 per 2-pack (~$69.44 per light)

- **Quantity:** 2 packs = 4 lights total

- **Use Case:** Indoor alert lighting for rooms, offices, display areas

- **Features:** White & Color Ambiance LED, works with Alexa/Google/HomeKit

- **Installation:** Place behind displays or on walls for ambient alert lighting

Outdoor Lights - Philips Hue Discover Flood Light:

- **Product:** Philips Hue Discover Outdoor Smart Flood Light Fixture (2-Pack, Black)

- **Price:** ~$395.99 per 2-pack (~$198.00 per light)

- **Quantity:** 1 pack = 2 lights

- **Use Case:** Outdoor/exterior alert lighting for station building

- **Features:** 15W White and Color Ambiance LED, weatherproof, IP65 rated

- **Installation:** Mount on building exterior for visible outdoor alerts

Both light types:

- Require Philips Hue Bridge (sold separately, ~$50-60)

- Support full color spectrum (red for fire, blue for EMS)

- Controlled via Hue API from the alert system

- Can be grouped and controlled individually or together

─────────────────────────────────────────────────────────

### Option 3: HTTP-based Light Controller

Best for: Custom HTTP APIs, home automation systems, ESP8266/ESP32 projects

Hardware Needed:

- Any device that accepts HTTP POST requests to control lights

- Examples: ESP8266 with HTTP server, Home Assistant, OpenHAB, etc.

HTTP API Requirements:

Your light controller must accept POST requests to these endpoints:

1. /status`** (GET) - Health check

- Returns: `200 OK` if available

2. /set-color`** (POST) - Set solid color

- Body: `{"color": "red"|"blue"|"white"|"off", "alertType": "fire"|"ems"|"none"}`

- Returns: `200 OK` on success

3. /flash`** (POST) - Start flashing

- Body: `{"color": "red"|"blue", "alertType": "fire"|"ems", "duration": 30000}`

- Returns: `200 OK` on success

4. /stop`** (POST) - Stop flashing/turn off

- Body: `{}`

- Returns: `200 OK` on success

Example ESP8266 Code:

#include <ESP8266WiFi.h>
#include <ESP8266WebServer.h>
ESP8266WebServer server(80);
void setup() {
  Serial.begin(115200);
  // Configure WiFi
  WiFi.begin("YOUR_SSID", "YOUR_PASSWORD");
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }
  Serial.println("\nWiFi connected");
  Serial.println("IP address: " + WiFi.localIP().toString());
  // Configure LED pins
  pinMode(5, OUTPUT);  // Red
  pinMode(4, OUTPUT);   // Green
  pinMode(0, OUTPUT);   // Blue
  // Setup HTTP endpoints
  server.on("/status", handleStatus);
  server.on("/set-color", handleSetColor);
  server.on("/flash", handleFlash);
  server.on("/stop", handleStop);
  server.begin();
}
void loop() {
  server.handleClient();
}
void handleStatus() {
  server.send(200, "application/json", "{\"status\":\"ok\"}");
}
void handleSetColor() {
  String body = server.arg("plain");
  // Parse JSON and set color
  // Implementation depends on your JSON library
  server.send(200, "application/json", "{\"success\":true}");
}
void handleFlash() {
  // Start flashing
  server.send(200, "application/json", "{\"success\":true}");
}
void handleStop() {
  // Stop flashing
  digitalWrite(5, LOW);
  digitalWrite(4, LOW);
  digitalWrite(0, LOW);
  server.send(200, "application/json", "{\"success\":true}");
}

Configuration:

Create .env` file in `frontend/` directory:

VITE_LIGHT_TYPE=http
VITE_LIGHT_HTTP_URL=http://192.168.1.XXX:80
VITE_LIGHT_HTTP_API_KEY=optional-api-key-if-needed

─────────────────────────────────────────────────────────

## Option 4: Commercial Alert Lighting Systems

### Connecting Commercial Alert Lighting Systems

Many commercial alert lighting systems from Amazon or fire equipment suppliers can be integrated. See LIGHT_CONTROLLER_INTEGRATION.md` for detailed integration guide.

Quick Steps:

1. Identify your device type:

- USB Serial controlled → Use `VITE_LIGHT_TYPE=serial`

- HTTP API controlled → Use `VITE_LIGHT_TYPE=http`

- Other protocol → May need bridge service (see integration guide)

2. For USB Serial devices:

VITE_LIGHT_TYPE=serial

- Connect device via USB

- Browser will prompt to select port

- Device must accept: `COLOR:red`, `COLOR:blue`, `FLASH:red:500`, etc.

3. For HTTP API devices:

VITE_LIGHT_TYPE=http
   VITE_LIGHT_HTTP_URL=http://192.168.1.XXX:PORT

- Device must have HTTP endpoints for color/flash control

- See `LIGHT_CONTROLLER_INTEGRATION.md` for API requirements

4. For other protocols:

- Create a bridge service to translate protocols

- See `LIGHT_CONTROLLER_INTEGRATION.md` for examples

Need Help?

- Check product documentation for API/control interface

- Share product details (manufacturer, model, connection method)

- See `LIGHT_CONTROLLER_INTEGRATION.md` for detailed integration guide

─────────────────────────────────────────────────────────

## Display Configuration

### Room Displays vs Main Station Displays

The system supports two types of displays with different behaviors:

Room Displays:

- Light duration: **2 minutes** (120 seconds)

- Night mode: Can be enabled to dim dashboard during nighttime

- Use case: Individual rooms, sleeping quarters, offices

Main Station Displays:

- Light duration: **5 minutes** (300 seconds)

- Night mode: Disabled (always full brightness)

- Use case: Main station area, dispatch center, common areas

### Night Mode for Room Displays

Room displays can be configured with night mode to reduce brightness during nighttime hours (20:30 to 06:30).

How it works:

- **During nighttime (20:30 - 06:30):** Dashboard dims to 20% brightness

- **When alert arrives:** Dashboard automatically brightens to 100% brightness

- **After alert dismisses:** Dashboard dims back to 20% if still nighttime

- **During daytime:** Dashboard remains at full brightness

Benefits:

- Reduces light pollution in sleeping areas

- Less disruptive to sleeping personnel

- Automatically brightens when alerts arrive

- Smooth transitions (2-second fade)

Configuration:

Create .env` file in `frontend/` directory:

# For room displays
VITE_DISPLAY_TYPE=room
VITE_NIGHT_MODE_ENABLED=true
# For main station displays
VITE_DISPLAY_TYPE=main-station
# Night mode is automatically disabled for main station displays

Or configure via browser localStorage:

// For room displays with night mode
localStorage.setItem('displayType', 'room')
localStorage.setItem('nightModeEnabled', 'true')
// For main station displays
localStorage.setItem('displayType', 'main-station')
// Then reload the page
location.reload()

─────────────────────────────────────────────────────────

## Nighttime Mode

### Automatic Gradual Fade-In (20:30 - 06:30)

To avoid startling sleeping personnel, the system automatically uses a gradual fade-in for lights during nighttime hours (20:30 to 06:30).

How it works:

- **Nighttime (20:30 - 06:30):** Lights fade in from 0% to 100% brightness over 5 seconds, then remain solid (not flashing)

- **Daytime (06:30 - 20:30):** Lights flash immediately as normal

Benefits:

- Prevents sudden bright flashes that could wake sleeping firefighters

- Provides gentle visual alert that gradually increases in intensity

- Lights remain solid (not flashing) during nighttime to avoid disturbance

- Automatic - no configuration needed

Technical Details:

- Fade duration: 5 seconds (5000ms)

- Brightness steps: 20 increments for smooth transition

- Time range: 20:30 (8:30 PM) to 06:30 (6:30 AM)

- Works with all light controller types (Serial, HTTP, Philips Hue)

Note: The system automatically detects nighttime based on the computer's system clock. Ensure your computer's time is set correctly.

─────────────────────────────────────────────────────────

## Configuration

### Method 1: Environment Variables (Recommended)

Create a .env` file in the `frontend/` directory:

For Philips Hue:

VITE_LIGHT_TYPE=hue
VITE_HUE_BRIDGE_IP=192.168.1.100
VITE_HUE_USERNAME=abc123def456...
VITE_HUE_LIGHT_IDS=1,2,3

For HTTP Controller:

VITE_LIGHT_TYPE=http
VITE_LIGHT_HTTP_URL=http://192.168.1.100:8080
VITE_LIGHT_HTTP_API_KEY=your-api-key

For Serial (Arduino):

VITE_LIGHT_TYPE=serial

After creating .env`, restart the frontend dev server.

### Method 2: Browser localStorage

You can also configure lights via browser console:

// For Philips Hue
localStorage.setItem('lightType', 'hue')
localStorage.setItem('hueBridgeIp', '192.168.1.100')
localStorage.setItem('hueUsername', 'abc123def456...')
localStorage.setItem('hueLightIds', '1,2,3')
// For HTTP Controller
localStorage.setItem('lightType', 'http')
localStorage.setItem('lightHttpUrl', 'http://192.168.1.100:8080')
localStorage.setItem('lightHttpApiKey', 'your-api-key')
// For Serial
localStorage.setItem('lightType', 'serial')
// Then reload the page
location.reload()

─────────────────────────────────────────────────────────

## Testing

### Test Speakers

1. Ensure speakers are connected and set as default audio device

2. Open the dashboard and click "Start System" (if prompted)

3. Send a test alert:

# PowerShell
   Invoke-WebRequest -Uri http://localhost:3000/api/alert -Method POST -Body '{"call_type":"Structure Fire","address":"123 Test St","units":"Engine 1"}' -ContentType "application/json"

4. Verify:

- Beep sounds play through speakers

- TTS announcement plays through speakers

- Volume is audible throughout the station

### Test Lights

1. Ensure lights are configured (see Configuration section)

2. Send a test fire alert:

Invoke-WebRequest -Uri http://localhost:3000/api/alert -Method POST -Body '{"call_type":"Structure Fire","address":"123 Test St","units":"Engine 1"}' -ContentType "application/json"

3. Verify:

- **Daytime (06:30 - 20:30):** Lights flash RED for fire alerts, BLUE for EMS alerts

- **Nighttime (20:30 - 06:30):** Lights fade in from 0% to 100% over 5 seconds, then remain solid

- Lights turn off when alert is dismissed

### Test EMS Alert

Invoke-WebRequest -Uri http://localhost:3000/api/alert -Method POST -Body '{"call_type":"Medical Emergency","address":"456 Test Ave","units":"Ambulance 1"}' -ContentType "application/json"

Verify lights flash BLUE (daytime) or fade in BLUE (nighttime).

### Test Nighttime Mode

To test nighttime fade-in behavior:

1. Change your computer's system time to between 20:30 and 06:30

2. Send a test alert

3. Verify lights gradually fade in over 5 seconds (not instant flash)

4. Verify lights remain solid (not flashing) after fade-in completes

5. Reset system time when done testing

### Test Display Configuration

Test Room Display (2-minute lights):

1. Configure display as room:

localStorage.setItem('displayType', 'room')
   location.reload()

2. Send a test alert

3. Verify lights stay on for 2 minutes (not 5 minutes)

4. Verify alert auto-dismisses after 2 minutes

Test Main Station Display (5-minute lights):

1. Configure display as main station:

localStorage.setItem('displayType', 'main-station')
   location.reload()

2. Send a test alert

3. Verify lights stay on for 5 minutes

4. Verify alert auto-dismisses after 5 minutes

Test Room Display Night Mode:

1. Configure room display with night mode:

localStorage.setItem('displayType', 'room')
   localStorage.setItem('nightModeEnabled', 'true')
   location.reload()

2. Set system time to nighttime (20:30 - 06:30)

3. Verify dashboard dims to 20% brightness

4. Send a test alert

5. Verify dashboard brightens to 100% when alert arrives

6. Dismiss alert

7. Verify dashboard dims back to 20% after 2 seconds

8. Reset system time when done testing

─────────────────────────────────────────────────────────

## Troubleshooting

### Speakers Not Working

1. Check Windows audio settings:

- Right-click speaker icon → Sound settings

- Verify correct device is selected

- Check volume levels

2. Check browser permissions:

- Ensure autoplay is enabled

- Click "Start System" button on dashboard

- Check browser console for audio errors

3. Check audio cable connections:

- Verify speakers are powered on

- Check cable connections

- Try different audio output port

### Lights Not Working

1. Check configuration:

- Verify `.env` file exists and is correct

- Check browser console for errors

- Ensure lights are initialized (check console logs)

2. For Serial (Arduino):

- Verify Arduino is connected via USB

- Check COM port in Device Manager

- Ensure Arduino code is uploaded correctly

- Browser will prompt to select port on first use

3. For Philips Hue:

- Verify bridge IP address is correct

- Ensure bridge and computer are on same network

- Check username is valid (press bridge button and recreate if needed)

- Verify light IDs are correct

4. For HTTP Controller:

- Verify controller URL is accessible from browser

- Test endpoints manually with curl/Postman

- Check controller logs for errors

- Verify CORS is enabled if needed

### Lights Flash But Don't Stop

- Lights automatically stop when alert is dismissed

- If lights stay on, check browser console for errors

- Manually stop via browser console: `localStorage.clear()` then reload

─────────────────────────────────────────────────────────

## Safety Notes

- **Electrical Safety:** When wiring LEDs, ensure proper voltage/current ratings and use appropriate resistors

- **Fire Safety:** Use UL-listed LED strips and proper enclosures for fire station environments

- **Network Security:** If using network-connected lights, ensure proper firewall rules

- **Backup:** System will continue to function even if lights fail - lights are optional enhancement

─────────────────────────────────────────────────────────

## Support

For issues or questions:

1. Check browser console for error messages

2. Verify hardware connections

3. Test individual components (speakers/lights) separately

4. Review configuration settings

The alert system will continue to function normally even if speakers or lights are not configured - these are optional enhancements.