# MVFD Open Alert Fire Station Alert System

## Complete Installation & Setup Guide

Version: 1.0

Date: 2026

For: Contractors & Installers

─────────────────────────────────────────────────────────

## Table of Contents

1. [Pre-Installation Checklist](#pre-installation-checklist)

2. [Hardware Installation](#hardware-installation)

3. [Software Installation](#software-installation)

4. [Network Configuration](#network-configuration)

5. [System Configuration](#system-configuration)

6. [Testing & Verification](#testing--verification)

7. [Final Setup & Handoff](#final-setup--handoff)

8. [Troubleshooting](#troubleshooting)

─────────────────────────────────────────────────────────

## Pre-Installation Checklist

### Required Materials & Tools

Hardware (Verify All Items Received):

- [ ] 6x CanaKit Raspberry Pi 5 Starter Kit PRO (8GB RAM)

- [ ] 1x Amazon Fire TV 40" 2-Series

- [ ] 1x MAGEX 15.6" Touchscreen Monitor

- [ ] 2x HUABAN 1 Channel DC 5V Relay Module

- [ ] 1x SunFounder 2 Channel DC 5V Relay Module

- [ ] 1x 40pcs Female-to-Female Dupont Wire pack

- [ ] 1x Mygatti 14AWG Speaker Wire (300 FT)

- [ ] 2x Philips Hue Smart Play Light Bar Base Kit (2-Pack)

- [ ] 1x Philips Hue Discover Outdoor Smart Flood Light (2-Pack)

- [ ] 1x Philips Hue Bridge (if not already owned)

Tools Required:

- [ ] Phillips screwdriver set

- [ ] Wire strippers

- [ ] Wire cutters

- [ ] Electrical tape

- [ ] Cable ties/zip ties

- [ ] Drill & bits (for mounting)

- [ ] Level

- [ ] Measuring tape

- [ ] Network cable tester

- [ ] Multimeter (for testing relays)

Software & Access:

- [ ] Computer/laptop with internet access

- [ ] MicroSD card reader (if not included with Raspberry Pi kits)

- [ ] USB keyboard and mouse (for initial Raspberry Pi setup)

- [ ] HDMI cable (for initial Raspberry Pi setup)

- [ ] Network access to station's router/switch

- [ ] Admin access to network router (for static IP configuration)

### Site Survey

Before Starting Installation:

1. Display Locations:

- [ ] Identify main station display location (Fire TV 40")

- [ ] Identify room display locations (MAGEX touchscreens)

- [ ] Verify power outlets available at each location

- [ ] Verify network connectivity (Ethernet or WiFi) at each location

- [ ] Measure distances for cable runs

2. Speaker Locations:

- [ ] Identify room speaker locations

- [ ] Verify power for speakers/amplifiers

- [ ] Plan speaker wire routing paths

- [ ] Measure speaker wire distances

3. Relay Installation:

- [ ] Identify Raspberry Pi location (central location preferred)

- [ ] Verify GPIO pin access for relay connections

- [ ] Plan relay module mounting locations

- [ ] Verify 5V power availability for relays

4. Lighting Locations:

- [ ] Identify indoor light locations (Hue Play Light Bars)

- [ ] Identify outdoor light locations (Hue Discover Flood Lights)

- [ ] Verify power outlets for lights

- [ ] Verify Hue Bridge location (needs Ethernet connection)

5. Network Infrastructure:

- [ ] Verify router/switch capacity

- [ ] Identify available network ports

- [ ] Plan IP address scheme

- [ ] Verify WiFi coverage (if using WiFi)

─────────────────────────────────────────────────────────

## Hardware Installation

### Phase 1: Display Installation

#### Step 1.1: Install Main Station Display (Fire TV)

1. Mount Fire TV:

- Choose location with good visibility

- Use TV wall mount (if provided) or place on stand

- Ensure power outlet nearby

- Ensure HDMI port accessible

2. Connect Raspberry Pi 5:

- Connect HDMI cable from Raspberry Pi 5 to Fire TV HDMI input

- Connect Raspberry Pi power supply

- Connect Ethernet cable (if using wired network)

- Power on Raspberry Pi

3. Label Connections:

- Label HDMI cable: "Main Station Display"

- Label Ethernet cable: "Main Station Pi"

- Note which HDMI input on TV (e.g., HDMI 1)

Checkpoint: Fire TV displays Raspberry Pi desktop when powered on.

#### Step 1.2: Install Room Displays (MAGEX Touchscreen)

For Each Room Display:

1. Mount Monitor:

- Use VESA mount or provided stand

- Position for easy viewing and touch access

- Ensure power outlet nearby

- Ensure HDMI port accessible

2. Connect Raspberry Pi 5:

- Connect HDMI cable from Raspberry Pi 5 to monitor HDMI input

- Connect Raspberry Pi power supply

- Connect Ethernet cable (if using wired network)

- Power on Raspberry Pi

3. Label Connections:

- Label HDMI cable: "Room Display - [Room Name]"

- Label Ethernet cable: "Room Pi - [Room Name]"

- Create room identification label on monitor

Checkpoint: Each MAGEX monitor displays Raspberry Pi desktop when powered on.

### Phase 2: Raspberry Pi Initial Setup

For Each Raspberry Pi (Backend + All Displays):

#### Step 2.1: Install Operating System

1. Download Raspberry Pi OS:

- Go to: https://www.raspberrypi.com/software/

- Download Raspberry Pi Imager

- Install Raspberry Pi Imager on your computer

2. Flash microSD Card:

- Insert microSD card into computer

- Open Raspberry Pi Imager

- Select "Raspberry Pi OS (64-bit)" with desktop

- Select your microSD card

- Click "Write" and wait for completion

3. Configure Before First Boot:

- Click gear icon in Raspberry Pi Imager

- Enable SSH

- Set username: `pi` (or custom)

- Set password: `[Secure Password]`

- Set WiFi credentials (if using WiFi)

- Set locale settings

- Click "Save" and write to card

4. Insert Card and Boot:

- Insert microSD card into Raspberry Pi

- Connect keyboard, mouse, and monitor (for first boot)

- Power on Raspberry Pi

- Complete initial setup wizard

#### Step 2.2: Update System

# Connect via SSH or use desktop terminal sudo apt update sudo apt upgrade -y sudo reboot

#### Step 2.3: Install Required Software

# Install Node.js (LTS version) curl -fsSL https://deb.nodesource.com/setup_lts.x | sudo -E bash - sudo apt install -y nodejs  # Verify installation node --version  # Should show v20.x or higher npm --version   # Should show 10.x or higher  # Install PM2 (process manager) sudo npm install -g pm2  # Install Git (if not already installed) sudo apt install -y git

Checkpoint: All Raspberry Pis have Node.js and PM2 installed.

### Phase 3: Relay Module Installation

#### Step 3.1: Identify GPIO Pin Assignments

System Reserved Pins:

- GPIO 18: Amplifier relay (if used)

- GPIO 23: Radio relay (if used)

Unit Pin Assignments (Auto-assigned):

- GPIO 4: Engine 2

- GPIO 5: Tanker 2

- GPIO 6: Tanker 21

- GPIO 7: Squad 2

- GPIO 8: Brush 2

- GPIO 9: Response 2

- GPIO 21: Medic 21 / Ambulance 21 (shared)

- GPIO 22: Medic 22 / Ambulance 22 (shared)

#### Step 3.2: Wire Relay Modules

For Each Room Speaker Relay:

1. Connect Power:

Relay VCC → Raspberry Pi Pin 2 (5V)    Relay GND → Raspberry Pi Pin 6 (GND)

2. Connect Signal:

Relay IN → Raspberry Pi GPIO Pin (assigned to unit)

- Use Female-to-Female Dupont wires

- Use different colors for easy identification

- Label each wire with unit name

3. Mount Relay Module:

- Secure relay module near Raspberry Pi

- Use double-sided tape or screws

- Ensure good ventilation

- Keep wires organized with cable ties

4. Test Relay:

# Install GPIO testing tool    sudo apt install -y gpiod        # Test relay (replace PIN with actual GPIO number)    gpioset gpiochip0 PIN=1  # Should activate relay (LED lights)    gpioset gpiochip0 PIN=0  # Should deactivate relay

Checkpoint: All relays activate/deactivate when GPIO pins are toggled.

#### Step 3.3: Connect Speakers to Relays

For Each Room:

1. Identify Speaker Wiring:

- Determine if controlling power or audio signal

- Most common: Control speaker power supply

2. Wire Relay to Speaker:

Speaker Power Supply + → Relay COM    Speaker Input + → Relay NO (Normally Open)

- When relay activates (GPIO LOW), power is interrupted (speaker muted)

- When relay deactivates (GPIO HIGH), power flows (speaker active)

3. Run Speaker Wire:

- Use 14AWG speaker wire for runs

- Keep wire runs as short as possible

- Avoid running near power lines

- Use cable ties to secure wire

- Label wire ends with room name

Checkpoint: Speakers mute/unmute when relays are activated.

### Phase 4: Lighting Installation

#### Step 4.1: Set Up Philips Hue Bridge

1. Connect Hue Bridge:

- Connect Ethernet cable from router to Hue Bridge

- Connect power adapter to Hue Bridge

- Wait for all LEDs to be solid (indicates ready)

2. Find Bridge IP Address:

- Check router admin panel for connected devices

- Or use Hue app to discover bridge

- Note IP address (e.g., 192.168.1.50)

3. Create API Username:

# Press button on Hue Bridge, then within 30 seconds:    curl -X POST http://[BRIDGE_IP]/api \      -H "Content-Type: application/json" \      -d '{"devicetype":"FireStation#AlertSystem"}'

- Copy the `username` from response

- Save for later configuration

Checkpoint: Hue Bridge responds to API requests.

#### Step 4.2: Install Indoor Lights (Hue Play Light Bars)

For Each Light Bar:

1. Mount Light Bar:

- Position behind displays or on walls

- Use included mounting hardware

- Ensure power outlet nearby

- Connect power adapter

2. Add to Hue System:

- Open Hue app on phone

- Go to Settings → Lights

- Press "Add light"

- Follow app instructions to pair light

- Note light ID (usually 1, 2, 3, etc.)

3. Test Light:

# Test light control (replace USERNAME and LIGHT_ID)    curl -X PUT http://[BRIDGE_IP]/api/[USERNAME]/lights/[LIGHT_ID]/state \      -H "Content-Type: application/json" \      -d '{"on":true,"bri":254,"hue":0,"sat":254}'

- Light should turn red

- Change hue to 46920 for blue

Checkpoint: All indoor lights respond to API commands.

#### Step 4.3: Install Outdoor Lights (Hue Discover Flood Lights)

For Each Flood Light:

1. Mount Flood Light:

- Choose exterior location with good visibility

- Use weatherproof mounting hardware

- Ensure power outlet nearby (weatherproof outlet recommended)

- Connect power adapter

2. Add to Hue System:

- Open Hue app

- Add light following same process as indoor lights

- Note light ID

3. Test Light:

- Use same API test as indoor lights

- Verify weatherproofing is secure

Checkpoint: All outdoor lights respond to API commands.

─────────────────────────────────────────────────────────

## Software Installation

### Phase 5: Backend Server Setup

#### Step 5.1: Install Backend Software

On Backend Raspberry Pi:

1. Clone or Copy Project:

cd ~    git clone [PROJECT_REPO_URL]    # OR copy project files via USB/SFTP    cd MVFD-Phoenix

2. Install Dependencies:

npm install

3. Configure Environment:

nano .env

Add configuration:

PORT=3000    API_KEY=[Generate secure random key]    NODE_ENV=production        # Room Speaker Configuration    ROOM_SPEAKERS=engine_bay:Engine 2|Tanker 2,office:Medic 21|Medic 22,dorm:Tanker 21|Squad 2,kitchen:,conference:        # Optional: Discord/Slack webhooks    DISCORD_WEBHOOK_URL=    SLACK_WEBHOOK_URL=

4. Build Frontend:

cd frontend    npm install    npm run build    cd ..

5. Set Static IP Address:

sudo nano /etc/dhcpcd.conf

Add:

interface eth0    static ip_address=192.168.1.100/24    static routers=192.168.1.1    static domain_name_servers=192.168.1.1 8.8.8.8

Replace IP addresses with your network settings.

6. Start Backend Service:

pm2 start npm --name "mvfd-backend" -- start    pm2 save    pm2 startup    # Follow instructions to enable systemd service

Checkpoint: Backend server responds to http://[BACKEND_IP]:3000/health`

#### Step 5.2: Configure Firewall

# Allow port 3000 sudo ufw allow 3000/tcp sudo ufw enable

### Phase 6: Frontend Display Setup

For Each Display Raspberry Pi:

#### Step 6.1: Install Frontend Software

1. Copy Project Files:

cd ~    # Copy project files (same as backend)    cd MVFD-Phoenix/frontend

2. Install Dependencies:

npm install

3. Configure Environment:

nano .env

For Main Station Display:

VITE_BACKEND_URL=http://192.168.1.100:3000    VITE_DISPLAY_TYPE=main-station    VITE_NIGHT_MODE_ENABLED=false

For Room Displays:

VITE_BACKEND_URL=http://192.168.1.100:3000    VITE_DISPLAY_TYPE=room    VITE_NIGHT_MODE_ENABLED=true    VITE_ROOM_ID=[room_name]    VITE_ROOM_NAME=[Room Display Name]

4. Build Frontend:

npm run build

#### Step 6.2: Set Up Kiosk Mode

1. Install Chromium:

sudo apt install -y chromium-browser

2. Create Autostart Script:

mkdir -p ~/.config/autostart    nano ~/.config/autostart/kiosk.desktop

Add:

[Desktop Entry]    Type=Application    Name=MVFD Phoenix Kiosk    Exec=chromium-browser --kiosk --autoplay-policy=no-user-gesture-required --disable-infobars http://192.168.1.100:3000

3. Disable Screen Saver:

sudo apt install -y xscreensaver    # Disable screensaver in desktop settings

Checkpoint: Display automatically opens browser in kiosk mode on boot.

─────────────────────────────────────────────────────────

## Network Configuration

### Phase 7: Network Setup

#### Step 7.1: Configure Static IPs

For Each Raspberry Pi:

1. Set Static IP:

sudo nano /etc/dhcpcd.conf

Add (adjust for each device):

interface eth0    static ip_address=192.168.1.101/24  # Backend: .100, Displays: .101, .102, etc.    static routers=192.168.1.1    static domain_name_servers=192.168.1.1 8.8.8.8

2. Reboot:

sudo reboot

#### Step 7.2: Verify Network Connectivity

From Each Display, Test Backend:

ping 192.168.1.100 curl http://192.168.1.100:3000/health

Checkpoint: All devices can ping and access backend server.

─────────────────────────────────────────────────────────

## System Configuration

### Phase 8: Configure Station Units

1. Access Admin Panel:

- Open browser: `http://192.168.1.100:3000/#station-units`

- Login with admin password (set in backend `.env`)

2. Add Station Units:

- Click "Add Unit"

- Enter unit names:

- Medic 22

- Medic 21

- Ambulance 21

- Ambulance 22

- Engine 2

- Tanker 2

- Tanker 21

- Squad 2

- Brush 2

- Response 2

- Save each unit

3. Verify GPIO Pin Assignments:

- Access: `http://192.168.1.100:3000/api/unit-pins`

- Verify pins are assigned correctly

### Phase 9: Configure Room Speakers

For Each Room:

1. Access Room Speaker Admin:

- Open: `http://192.168.1.100:3000/#room-speaker`

- Login with admin password

2. Configure Room:

- Set Room ID (e.g., `engine_bay`)

- Set Room Name (e.g., `Engine Bay`)

- Select units for this room

- Save configuration

3. Or Configure via Main Screen:

- On room display, click "Select Units" button

- Select units from popup

- Click "Save"

### Phase 10: Configure Lighting

1. Get Hue Light IDs:

curl http://[BRIDGE_IP]/api/[USERNAME]/lights

- Note all light IDs

2. Configure Frontend:

localStorage.setItem('lightType', 'hue')    localStorage.setItem('hueBridgeIP', '[BRIDGE_IP]')    localStorage.setItem('hueUsername', '[USERNAME]')    localStorage.setItem('hueLightIds', '1,2,3,4,5,6')  // All light IDs

- On each display, configure via browser localStorage or `.env`:

─────────────────────────────────────────────────────────

## Testing & Verification

### Phase 11: System Testing

#### Test 1: Backend Health

curl http://192.168.1.100:3000/health

Expected: {"status":"ok","message":"OpenPhoenix API is running"}`

#### Test 2: Socket.io Connection

1. Open browser on any display

2. Open browser console (F12)

3. Look for: ✅ Connected to server`

#### Test 3: Relay Control

# Test each relay curl -X POST http://192.168.1.100:3000/api/unit-speaker/mute \   -H "Content-Type: application/json" \   -H "X-Admin-Token: [TOKEN]" \   -d '{"units":["Engine 2"],"mute":false}'

Expected: Speaker unmutes for Engine 2

#### Test 4: Alert Propagation

curl -X POST http://192.168.1.100:3000/api/alert \   -H "Content-Type: application/json" \   -H "X-API-Key: [API_KEY]" \   -d '{     "call_type": "Structure Fire",     "address": "123 Test St",     "units": "Engine 2",     "narrative": "Test alert"   }'

Expected:

- Alert appears on all displays

- Lights flash red

- Sound plays

- Room speakers activate for matching units

#### Test 5: Station Alert

curl -X POST http://192.168.1.100:3000/api/alert \   -H "Content-Type: application/json" \   -H "X-API-Key: [API_KEY]" \   -d '{     "call_type": "Station Alert",     "address": "Station",     "units": "Station",     "narrative": "Test station alert"   }'

Expected: Alert plays in ALL rooms (all speakers activate)

#### Test 6: Quiet Mode

1. On room display, click "Quiet Mode" button

2. Send test alert

3. Expected: Alert shows on screen but no audio plays

4. Disable quiet mode

5. Expected: Audio plays normally

#### Test 7: Unit Selection

1. On room display, click "Select Units"

2. Select specific units (e.g., Engine 2, Tanker 2)

3. Save

4. Send alert with Engine 2

5. Expected: Alert plays in this room

6. Send alert with Medic 21

7. Expected: Alert does NOT play in this room

─────────────────────────────────────────────────────────

## Final Setup & Handoff

### Phase 12: Documentation & Training

1. Create System Documentation:

- Document all IP addresses

- Document all GPIO pin assignments

- Document room configurations

- Document admin passwords (store securely)

2. Create Quick Reference Card:

- Backend IP: `192.168.1.100`

- Admin URL: `http://192.168.1.100:3000/#station-units`

- API Key: `[Store securely]`

- Admin Password: `[Store securely]`

3. Train Station Personnel:

- How to access admin panels

- How to configure room units

- How to enable/disable quiet mode

- How to test system

- How to restart services

4. Provide Maintenance Information:

- How to restart backend: `pm2 restart mvfd-backend`

- How to view logs: `pm2 logs mvfd-backend`

- How to update software

- Contact information for support

### Phase 13: Final Checklist

- [ ] All displays show dashboard correctly

- [ ] All displays connect to backend

- [ ] Alerts appear on all displays simultaneously

- [ ] Room speakers activate for assigned units

- [ ] Station alerts play in all rooms

- [ ] Quiet mode works on all room displays

- [ ] Unit selection works on all room displays

- [ ] Lights flash correctly (red for fire, blue for EMS)

- [ ] Sound alerts play correctly

- [ ] All relays respond to GPIO commands

- [ ] Network connectivity verified

- [ ] Static IPs configured

- [ ] Firewall rules configured

- [ ] Kiosk mode enabled on all displays

- [ ] Auto-start configured on all Raspberry Pis

- [ ] Documentation completed

- [ ] Personnel trained

─────────────────────────────────────────────────────────

## Troubleshooting

### Common Issues

#### Display Not Showing Dashboard

Symptoms: Blank screen or error message

Solutions:

1. Check backend is running: curl http://192.168.1.100:3000/health`

2. Check network connectivity: ping 192.168.1.100`

3. Check browser console for errors (F12)

4. Verify .env` file has correct `VITE_BACKEND_URL`

5. Rebuild frontend: npm run build`

#### Room Speakers Not Activating

Symptoms: Alerts play but room speakers don't activate

Solutions:

1. Check relay wiring (VCC, GND, IN connections)

2. Test relay manually: gpioset gpiochip0 PIN=0`

3. Check GPIO pin assignment: curl http://192.168.1.100:3000/api/unit-pins`

4. Verify room unit configuration

5. Check relay LED (should light when activated)

#### Lights Not Flashing

Symptoms: Alerts play but lights don't flash

Solutions:

1. Check Hue Bridge connection (Ethernet)

2. Verify Hue Bridge IP address

3. Test Hue API: curl http://[BRIDGE_IP]/api/[USERNAME]/lights`

4. Verify light IDs in configuration

5. Check browser console for Hue API errors

#### Alerts Not Appearing on All Displays

Symptoms: Alert appears on some displays but not others

Solutions:

1. Check Socket.io connection on each display (browser console)

2. Verify all displays have correct VITE_BACKEND_URL`

3. Check backend logs: pm2 logs mvfd-backend`

4. Verify network connectivity from each display

5. Check firewall rules allow port 3000

#### Raspberry Pi Won't Boot

Symptoms: No display, no network activity

Solutions:

1. Check power supply (use official Raspberry Pi power supply)

2. Check microSD card (try re-flashing)

3. Check HDMI connection

4. Try different microSD card

5. Check for physical damage

### Getting Help

Logs to Collect:

# Backend logs pm2 logs mvfd-backend --lines 100  # System logs journalctl -u pm2-mvfd-backend -n 100  # Network status ip addr show ping 192.168.1.100

Information to Provide:

- Error messages from browser console

- Backend log output

- Network configuration

- GPIO pin assignments

- Room configurations

─────────────────────────────────────────────────────────

## Appendix

### GPIO Pin Reference

Physical Pin Layout (40-pin header):

3.3V  [1]  [2]  5V    GPIO2  [3]  [4]  5V    GPIO3  [5]  [6]  GND    GPIO4  [7]  [8]  GPIO14     GND  [9] [10]  GPIO15   GPIO17 [11] [12] GPIO18  ← Amplifier   GPIO27 [13] [14] GND   GPIO22 [15] [16] GPIO23  ← Radio     3.3V [17] [18] GPIO24   GPIO10 [19] [20] GND    GPIO9 [21] [22] GPIO25   GPIO11 [23] [24] GPIO8     GND [25] [26] GPIO7    GPIO0 [27] [28] GPIO1    GPIO5 [29] [30] GND    GPIO6 [31] [32] GPIO12   GPIO13 [33] [34] GND   GPIO19 [35] [36] GPIO16   GPIO26 [37] [38] GPIO20     GND [39] [40] GPIO21

### Network Configuration Reference

Default IP Scheme:

- Backend Server: `192.168.1.100`

- Display 1 (Main Station): `192.168.1.101`

- Display 2 (Room 1): `192.168.1.102`

- Display 3 (Room 2): `192.168.1.103`

- Hue Bridge: `192.168.1.50` (or DHCP assigned)

Ports Used:

- `3000`: Backend HTTP/Socket.io server

- `80`: Hue Bridge HTTP API (if configured)

### Quick Command Reference

# Backend management pm2 start mvfd-backend pm2 stop mvfd-backend pm2 restart mvfd-backend pm2 logs mvfd-backend  # Test backend curl http://192.168.1.100:3000/health  # Test alert curl -X POST http://192.168.1.100:3000/api/alert \   -H "Content-Type: application/json" \   -H "X-API-Key: [KEY]" \   -d '{"call_type":"Test","address":"123 Test","units":"Engine 2"}'  # Test GPIO gpioset gpiochip0 PIN=0  # Activate relay gpioset gpiochip0 PIN=1  # Deactivate relay  # Test Hue curl http://[BRIDGE_IP]/api/[USERNAME]/lights

─────────────────────────────────────────────────────────

End of Installation Guide

For additional support, refer to:

- `ROOM_SPEAKER_SETUP.md` - Room speaker configuration details

- `MULTI_DISPLAY_SETUP.md` - Multi-display architecture details

- `HARDWARE_SETUP.md` - Hardware setup details

- `PRODUCT_INVENTORY.md` - Complete product list