# Light Controller Integration Guide

Guide for connecting various alert lighting systems to the Fire Station Alert System.

## Currently Supported Interfaces

The system supports three types of light controllers:

1. USB Serial - Arduino-based or serial-controlled lights

2. HTTP/REST API - Network-enabled devices with HTTP endpoints

3. Philips Hue - Smart home lighting via Hue Bridge

## How to Determine Your Device Type

To connect your Amazon alert lighting system, please check:

### 1. Check Product Documentation

Look for:

- **Connection method:** USB, Ethernet, WiFi, Serial Port

- **Control protocol:** HTTP API, Serial commands, TCP/IP, Proprietary

- **Software/App:** Does it come with control software or an app?

- **API Documentation:** Check manufacturer's website for API docs

### 2. Common Alert Lighting System Types

Type A: USB Serial Controlled

- Connects via USB cable

- Uses serial commands (like Arduino)

- Usually controlled by simple text commands

- ✅ **Already Supported** - Use `VITE_LIGHT_TYPE=serial`

Type B: HTTP API Controlled

- Connects via network (Ethernet/WiFi)

- Has an IP address

- Controlled via HTTP POST/GET requests

- ✅ **Already Supported** - Use `VITE_LIGHT_TYPE=http`

Type C: TCP/IP Socket Controlled

- Connects via network

- Uses raw TCP/IP sockets

- May need custom integration

- ⚠️ **May need custom code** - See below

Type D: Proprietary Protocol

- Uses manufacturer-specific protocol

- May require SDK or special drivers

- ⚠️ **Needs custom integration** - See below

## Integration Steps

### Step 1: Identify Your Device

Please provide:

1. Product name/model number

2. Connection method (USB, Ethernet, WiFi)

3. Control interface (Serial, HTTP API, TCP/IP, Proprietary)

4. API documentation (if available)

### Step 2: Choose Integration Method

#### Method 1: USB Serial (If Supported)

If your device accepts serial commands:

1. Configure:

VITE_LIGHT_TYPE=serial

2. Connect device via USB

3. Browser will prompt to select serial port

4. Device must accept these commands:

- `COLOR:red` - Set red color

- `COLOR:blue` - Set blue color

- `COLOR:off` - Turn off

- `FLASH:red:500` - Flash red every 500ms

- `FADEIN:red:5000` - Fade in red over 5 seconds

Note: If your device uses different commands, you may need to create an Arduino intermediary or modify the serial controller code.

#### Method 2: HTTP API (If Supported)

If your device has an HTTP API:

1. Configure:

VITE_LIGHT_TYPE=http
   VITE_LIGHT_HTTP_URL=http://192.168.1.XXX:PORT
   VITE_LIGHT_HTTP_API_KEY=your-api-key-if-needed

2. Your device must accept HTTP POST requests to:

- `/status` (GET) - Health check

- `/set-color` (POST) - Set color: `{"color": "red", "alertType": "fire"}`

- `/flash` (POST) - Flash: `{"color": "red", "alertType": "fire", "duration": 30000}`

- `/fade-in` (POST) - Fade in: `{"color": "red", "alertType": "fire", "fadeDuration": 5000}`

- `/stop` (POST) - Stop: `{}`

Note: If your device uses different endpoints or request format, you may need to create a bridge service or modify the HTTP controller code.

#### Method 3: Custom Integration

If your device uses a different protocol, you can:

1. Create a bridge service (recommended)

2. Add custom controller class to the codebase

3. Use existing HTTP controller with a bridge

## Creating a Bridge Service

If your device doesn't match existing interfaces, create a simple bridge:

### Example: Bridge Service (Node.js)

Create a bridge that translates our HTTP API to your device's protocol:

// bridge-service.js
const express = require('express');
const app = express();
app.use(express.json());
// Translate our API to your device's protocol
app.post('/set-color', async (req, res) => {
  const { color, alertType } = req.body;
  // Call your device's API here
  // Example: await yourDevice.setColor(color);
  res.json({ success: true });
});
app.post('/flash', async (req, res) => {
  const { color, alertType, duration } = req.body;
  // Implement flashing logic for your device
  // Example: await yourDevice.flash(color, duration);
  res.json({ success: true });
});
app.listen(8080, () => {
  console.log('Bridge service running on port 8080');
});

Then configure frontend to use the bridge:

VITE_LIGHT_TYPE=http
VITE_LIGHT_HTTP_URL=http://localhost:8080

## Common Alert Lighting Systems

### Whelen Engineering Systems

- Usually use serial or proprietary protocols

- May need bridge service

- Check manufacturer documentation

### Federal Signal Systems

- Often use serial or network protocols

- May have HTTP API available

- Check product documentation

### Code 3 Systems

- Various interfaces depending on model

- Check specific product documentation

### Generic LED Controllers

- Many use USB serial (Arduino-compatible)

- Usually work with serial controller

- May need command translation

## Getting Help

To add support for your specific device, please provide:

1. Product Information:

- Manufacturer name

- Model number

- Product link or documentation

2. Technical Details:

- Connection method (USB, Ethernet, WiFi)

- Control protocol (Serial, HTTP, TCP/IP, Proprietary)

- API documentation or command reference

- Example commands/code (if available)

3. Current Setup:

- How do you currently control it?

- What software/app does it use?

- Any existing integration examples?

## Quick Test

To test if your device might work:

1. For USB Serial devices:

- Connect via USB

- Try configuring: `VITE_LIGHT_TYPE=serial`

- Browser will prompt for port selection

- Check browser console for connection status

2. For Network devices:

- Find device IP address

- Try accessing: `http://DEVICE_IP/status` or similar

- Check device documentation for API endpoints

- Configure: `VITE_LIGHT_TYPE=http`

3. Check device documentation:

- Look for "API", "Integration", "Developer", or "SDK" sections

- Check for HTTP endpoints or serial command references

- Look for example code or integration guides

## Next Steps

1. Share product details - Manufacturer, model, connection method

2. Check documentation - Look for API or control interface info

3. Test existing interfaces - Try serial or HTTP if applicable

4. Create bridge if needed - Simple service to translate protocols

Once you provide the product details, I can help create a custom integration or guide you through the setup process!