# Room Speaker Control Setup Guide

Guide for setting up individual room speaker control with unit-based dispatch routing and quiet mode.

## Frontend Control Panel

Access the Room Speaker Admin Panel:

- Navigate to `#room-speaker` in your browser (e.g., `http://localhost:5173/#room-speaker`)

- Or click the "Speakers" link in the bottom-left corner of the dashboard

Features:

- ✅ **Select Units:** Choose which units trigger alerts in this room (via admin panel or main screen popup)

- ✅ **Configure Room:** Set room ID, name, and backend URL

- ✅ **Quiet Mode Toggle:** Mute/unmute this room with one click (button on main screen)

- ✅ **Control All Rooms:** Mute/unmute any room from the frontend

- ✅ **View Status:** See GPIO pin, assigned units, and relay status

Main Screen Controls (Room Displays):

- **Unit Selector Button:** Bottom-left corner, below radar - Opens popup to select units for this room

- **Quiet Mode Button:** Bottom-left corner, below unit selector - Toggle quiet mode on/off

- Both buttons only appear when room speaker is configured

## Overview

This feature allows each room to have its own relay-controlled speaker system with:

- **Unit-Based Routing:** Each room only plays alerts for assigned units

- **Quiet Mode:** Mute individual rooms for meetings/events

- **Independent Control:** Each room speaker controlled separately via GPIO relay

## Use Cases

1. Unit-Based Dispatches:

- Room 1 (Engine Bay): Only plays alerts for "Engine 1", "Engine 2"

- Room 2 (Office): Only plays alerts for "Medic 1", "Medic 2"

- Room 3 (Dorm): Only plays alerts for "Ladder 1", "Ladder 2"

2. Quiet Mode for Meetings:

- Mute office room during meetings

- Mute conference room during events

- Keep other rooms active

## Hardware Setup

### Components Needed Per Room

- **5V Relay Module** - One per room

- **HUABAN 1 Channel DC 5V Relay Module** (recommended)

- Low level trigger with optocoupler protection

- Includes LED indicator for relay status

- **Jumper Wires** - Female to Female Dupont wires for GPIO connections

- **40pcs Female to Female Dupont Wire** (20cm/8 inch)

- Multicolored for easy identification

- **Speaker Wire** - 14AWG speaker wire for audio connections

- **Mygatti 14AWG Speaker Wire** (300 FT roll, shared across rooms)

- 14/2 Gauge with polarity markers

- **Room Speaker/Amplifier** - Each room's speaker system

### Wiring Per Room

For Each Room:

1. Connect Relay to Raspberry Pi GPIO:

Relay VCC → Raspberry Pi Pin 2 (5V) [shared]
   Relay GND → Raspberry Pi Pin 6 (GND) [shared]
   Relay IN → Raspberry Pi GPIO Pin (unique per room)

Using HUABAN Relay Module:

- Connect VCC to Raspberry Pi 5V (Pin 2)

- Connect GND to Raspberry Pi GND (Pin 6)

- Connect IN (signal pin) to your assigned GPIO pin (e.g., GPIO 4 for Engine 2)

- Use Female-to-Female Dupont wires for connections

- **Note:** HUABAN modules use LOW level trigger (GPIO LOW = relay ON)

2. Connect Relay to Room Speaker:

Option A: Control Speaker Power (Recommended)

Speaker Power Supply + → Relay COM
   Speaker Power Input + → Relay NO
   When relay closes → Power interrupted → Speaker muted

Option B: Control Speaker Audio Signal

Audio Source → Relay COM
   Speaker Input → Relay NO
   When relay closes → Audio shorted → Speaker muted

### GPIO Pin Assignment (Unique Pins Per Unit)

GPIO pins are automatically assigned with unique pins for each unit:

Custom Pin Mapping:

- Each unit gets a unique GPIO pin

- Medic/Ambulance pairs share pins (same physical unit)

Example Pin Assignments:

- **Medic 21** / **Ambulance 21** → GPIO Pin **21** (shared)

- **Medic 22** / **Ambulance 22** → GPIO Pin **22** (shared)

- **Engine 2** → GPIO Pin **4** (unique)

- **Tanker 2** → GPIO Pin **5** (unique)

- **Tanker 21** → GPIO Pin **6** (unique)

- **Squad 2** → GPIO Pin **7** (unique)

- **Brush 2** → GPIO Pin **8** (unique)

- **Response 2** → GPIO Pin **9** (unique)

Key Points:

- Medic/Ambulance pairs share pins (e.g., "Medic 21" and "Ambulance 21" both use pin 21)

- All other units get unique pins

- Pins are assigned automatically when units are added to the system

- Reserved pins: GPIO 18 (amplifier), GPIO 23 (radio)

- Avoid pins: GPIO 2, 3 (I2C), GPIO 14, 15 (UART)

## Backend Configuration

### Step 1: Configure Room Speakers in Backend

Add to backend .env` file:

# Room Speaker Configuration
# Format: roomId:unit1|unit2|unit3
# GPIO pins are automatically assigned based on unit numbers
# Example: engine_bay:Engine 1|Engine 2|Ladder 1
ROOM_SPEAKERS=engine_bay:Engine 1|Engine 2|Ladder 1,office:Medic 1|Medic 2,dorm:Ladder 2|Engine 3,kitchen:

Format Explanation:

- `roomId`: Unique identifier (e.g., "engine_bay", "office")

- `units`: Pipe-separated list of units (optional - if empty, plays all alerts)

- **GPIO pins are automatically extracted from unit names** (see Pin Assignment below)

Examples:

Room with specific units:

engine_bay:Engine 1|Engine 2|Ladder 1

- Room ID: `engine_bay`

- Units: Only plays alerts for "Engine 1", "Engine 2", or "Ladder 1"

- GPIO Pins: Auto-assigned (Engine 1 → Pin 1, Engine 2 → Pin 2, Ladder 1 → Pin 1)

Room that plays all alerts:

kitchen:

- Room ID: `kitchen`

- Units: Empty (plays all alerts)

### Step 2: GPIO Pin Assignment (Automatic - Unique Pins)

GPIO pins are automatically assigned with unique pins for each unit:

Pin Assignment Rules:

- Each unit gets a unique GPIO pin

- Medic/Ambulance pairs share pins (same physical unit)

- Pins are assigned when units are added to the `station_units` database

Example Pin Assignments:

- **Medic 21** / **Ambulance 21** → GPIO Pin **21** (shared)

- **Medic 22** / **Ambulance 22** → GPIO Pin **22** (shared)

- **Engine 2** → GPIO Pin **4** (unique)

- **Tanker 2** → GPIO Pin **5** (unique)

- **Tanker 21** → GPIO Pin **6** (unique)

- **Squad 2** → GPIO Pin **7** (unique)

- **Brush 2** → GPIO Pin **8** (unique)

- **Response 2** → GPIO Pin **9** (unique)

Key Points:

- Medic/Ambulance pairs share pins (e.g., "Medic 21" and "Ambulance 21" both use pin 21)

- All other units get unique pins

- Reserved pins: GPIO 18 (amplifier), GPIO 23 (radio)

- Avoid pins: GPIO 2, 3 (I2C), GPIO 14, 15 (UART)

- Units are loaded from both `ROOM_SPEAKERS` env var and `station_units` database table

### Step 2: Install GPIO Library

On Raspberry Pi backend:

npm install onoff
npm install --save-dev @types/node

### Step 3: Restart Backend

pm2 restart mvfd-backend
# Or if using systemd:
sudo systemctl restart mvfd-phoenix

## Frontend Configuration

### Step 1: Configure Room Settings

Option A: Via Environment Variables

Add to frontend/.env`:

# Room Configuration
VITE_ROOM_ID=engine_bay
VITE_ROOM_NAME=Engine Bay
VITE_ROOM_UNITS=Engine 1,Engine 2,Ladder 1
VITE_BACKEND_URL=http://192.168.1.100:3000

Option B: Via Main Screen (Recommended)

1. Open the room display in a browser

2. Click the "Select Units" button (bottom-left, below radar)

3. Select units from the popup

4. Click "Save" - units are saved automatically

Option C: Via Admin Panel

1. Navigate to #room-speaker` admin page

2. Configure room ID, name, and select units

3. Save configuration

Option D: Via localStorage:

localStorage.setItem('roomId', 'engine_bay')
localStorage.setItem('roomName', 'Engine Bay')
localStorage.setItem('roomUnits', 'Engine 1,Engine 2,Ladder 1')
location.reload()

### Step 2: Rebuild Frontend (if using env vars)

cd frontend
npm run build

## How It Works

### Unit-Based Routing

When an alert arrives:

1. System checks alert units (e.g., "Engine 1, Ladder 2" or "Station")

2. If "Station" is alerted or no specific units → Play in ALL rooms ✅

3. If room has no units selected → Play all alerts ✅

4. Otherwise, compares with room's selected units

5. If match found → Unmute room speaker → Play alert

6. If no match → Keep room speaker muted → Skip alert

Alert Routing Rules:

- **Station Alert:** If alert contains "Station" → Plays in ALL rooms

- **No Units:** If alert has no specific units → Plays in ALL rooms

- **Room with No Units Selected:** If room has no units selected → Plays ALL alerts

- **Unit Match:** Alert only plays if units match room's selected units

Examples:

Example 1: Station Alert

- Alert: Units = "Station"

- Room 1 (Engine Bay): **Plays alert** ✅ (Station alert plays everywhere)

- Room 2 (Office): **Plays alert** ✅ (Station alert plays everywhere)

- Room 3 (Dorm): **Plays alert** ✅ (Station alert plays everywhere)

Example 2: Specific Unit Alert

- Alert: Units = "Engine 1, Medic 1"

- Room 1 (Engine Bay): Has "Engine 1" → **Plays alert** ✅

- Room 2 (Office): Has "Medic 1" → **Plays alert** ✅

- Room 3 (Dorm): Has "Ladder 2" → **Muted** 🔇

Example 3: Room with No Units Selected

- Alert: Units = "Engine 1, Medic 1"

- Room 1 (Engine Bay): No units selected → **Plays alert** ✅ (plays all alerts)

- Room 2 (Office): Has "Medic 1" → **Plays alert** ✅

- Room 3 (Dorm): Has "Ladder 2" → **Muted** 🔇

### Quiet Mode

Enable Quiet Mode:

- Mutes room speaker for meetings/events

- Alerts still show on screen, but no audio

- Can be toggled via UI or API

Quiet Mode Behavior:

- When enabled: All alerts muted (even matching units)

- When disabled: Normal unit-based routing resumes

- Persists across page reloads (stored in localStorage)

## API Endpoints

### Mute/Unmute Room Speaker

# Mute room speaker
curl -X POST http://localhost:3000/api/room-speaker/engine_bay/mute \
  -H "Content-Type: application/json" \
  -H "X-API-Key: your-api-key" \
  -d '{"mute": true}'
# Unmute room speaker
curl -X POST http://localhost:3000/api/room-speaker/engine_bay/mute \
  -H "Content-Type: application/json" \
  -H "X-API-Key: your-api-key" \
  -d '{"mute": false}'

### Get Room Status

curl http://localhost:3000/api/room-speaker/engine_bay/status \
  -H "X-API-Key: your-api-key"

### List All Rooms

curl http://localhost:3000/api/room-speakers \
  -H "X-API-Key: your-api-key"

## Frontend Controls

### Main Screen Unit Selection

On Room Displays:

- **Unit Selector Button:** Located bottom-left, below radar

- Click to open popup with all available units

- Select/deselect units with checkboxes

- Shows count of selected units on button

- Empty selection = plays all alerts

### Main Screen Quiet Mode

On Room Displays:

- **Quiet Mode Button:** Located bottom-left, below unit selector

- Toggle button with visual feedback

- Red when ON, gray when OFF

- Persists across page reloads

### Admin Panel Controls

Access via #room-speaker`:

- Configure room ID, name, backend URL

- Select units for this room

- Toggle quiet mode

- Control all rooms from one place

## Testing

### Test Unit-Based Routing

1. Configure room with specific units:

- Use main screen unit selector, or

- Set `VITE_ROOM_UNITS=Engine 1,Engine 2` in `.env`

2. Send test alert with matching unit:

curl -X POST http://localhost:3000/api/alert \
     -H "Content-Type: application/json" \
     -H "X-API-Key: your-api-key" \
     -d '{"call_type": "Structure Fire", "address": "123 Test St", "units": "Engine 1, Ladder 2"}'

- Should play alert (Engine 1 matches)

3. Send test alert with non-matching unit:

curl -X POST http://localhost:3000/api/alert \
     -H "Content-Type: application/json" \
     -H "X-API-Key: your-api-key" \
     -d '{"call_type": "Medical", "address": "456 Test St", "units": "Medic 1"}'

- Should NOT play alert (no matching units)

4. Send test Station alert (plays everywhere):

curl -X POST http://localhost:3000/api/alert \
     -H "Content-Type: application/json" \
     -H "X-API-Key: your-api-key" \
     -d '{"call_type": "Station Alert", "address": "789 Test St", "units": "Station"}'

- Should play alert in ALL rooms (Station alert)

5. Send test alert with no units (plays everywhere):

curl -X POST http://localhost:3000/api/alert \
     -H "Content-Type: application/json" \
     -H "X-API-Key: your-api-key" \
     -d '{"call_type": "General", "address": "999 Test St", "units": ""}'

- Should play alert in ALL rooms (no specific units)

### Test Quiet Mode

1. Enable quiet mode:

// In browser console
   localStorage.setItem('quietMode_engine_bay', 'true')
   location.reload()

2. Send test alert:

- Alert should show on screen

- No audio should play

3. Disable quiet mode:

localStorage.removeItem('quietMode_engine_bay')
   location.reload()

### Test Relay Control

# Test room speaker relay manually
gpio -g mode 24 out  # Replace 24 with your room's GPIO pin
gpio -g write 24 1   # Should mute speaker
gpio -g write 24 0   # Should unmute speaker

## Configuration Examples

### Example 1: Fire Station with Multiple Rooms

Backend .env`:**

# Room Speaker Configuration
# Format: roomId:unit1|unit2|unit3
# GPIO pins are auto-assigned based on unit names
ROOM_SPEAKERS=engine_bay:Engine 2|Tanker 2,office:Medic 21|Medic 22,dorm:Tanker 21|Squad 2,kitchen:,conference:

Room Configurations:

Engine Bay (Room 1):

- Room ID: `engine_bay`

- Units: Engine 2, Tanker 2 (selected via main screen or admin panel)

- GPIO Pins: 4 (Engine 2), 5 (Tanker 2)

Office (Room 2):

- Room ID: `office`

- Units: Medic 21, Medic 22 (selected via main screen or admin panel)

- GPIO Pins: 21 (Medic 21/Ambulance 21), 22 (Medic 22/Ambulance 22)

Dorm (Room 3):

- Room ID: `dorm`

- Units: Tanker 21, Squad 2 (selected via main screen or admin panel)

- GPIO Pins: 6 (Tanker 21), 7 (Squad 2)

Kitchen (Room 4):

- Room ID: `kitchen`

- Units: None selected (empty)

- Behavior: Plays ALL alerts (no unit filter)

Conference Room (Room 5):

- Room ID: `conference`

- Units: None selected (empty)

- Behavior: Plays ALL alerts (no unit filter)

- Can use quiet mode during meetings

Alert Behavior Examples:

Alert 1: Units = "Station"

- All rooms play alert ✅ (Station alert)

Alert 2: Units = "Engine 2"

- Engine Bay: Plays ✅ (matches Engine 2)

- Office: Muted 🔇 (no match)

- Dorm: Muted 🔇 (no match)

- Kitchen: Plays ✅ (no units selected = plays all)

- Conference: Plays ✅ (no units selected = plays all)

Alert 3: Units = "Medic 21"

- Engine Bay: Muted 🔇 (no match)

- Office: Plays ✅ (matches Medic 21)

- Dorm: Muted 🔇 (no match)

- Kitchen: Plays ✅ (no units selected = plays all)

- Conference: Plays ✅ (no units selected = plays all)

## Troubleshooting

### Room Speaker Not Muting

1. Check GPIO pin:

gpio -g read 24  # Replace with your GPIO pin

2. Check relay wiring:

- Verify relay connections

- Check relay LED (should light when activated)

3. Check room configuration:

curl http://localhost:3000/api/room-speakers -H "X-API-Key: your-api-key"

### Alerts Not Playing in Room

1. Check unit assignments:

- Verify room units match alert units

- Check unit matching logic (case-insensitive, partial match)

2. Check quiet mode:

// In browser console
   console.log(localStorage.getItem('quietMode_engine_bay'))

3. Check room configuration:

// In browser console
   import { getRoomConfig } from './utils/roomSpeakerController'
   console.log(getRoomConfig())

### Multiple Rooms Playing Same Alert

- This is normal if multiple rooms have matching units

- Each room independently decides whether to play

- To prevent, assign unique units to each room

## Parts List Per Room

What to Purchase Per Room:

1. 5V Relay Module - One per room

- **Recommended:** HUABAN 1 Channel DC 5V Relay Module with Optocoupler Low Level Trigger

- **Alternative:** SunFounder 2 Channel DC 5V Relay Module (if you need 2 rooms per module)

- **Price:** ~$6.50 per single-channel module, ~$6.79 per 2-channel module

2. Jumper Wires - For connecting relay to Raspberry Pi GPIO

- **Recommended:** 40pcs Female to Female Dupont Wire Breadboard Jumper Ribbon Cables (20cm/8 inch)

- **Price:** ~$3.99 for 40 pieces (enough for multiple rooms)

3. Speaker Wire - For connecting speakers to amplifiers

- **Recommended:** Mygatti 14AWG Speaker Wire (14/2 Gauge, 300 FT)

- **Price:** ~$45.99 for 300 feet (enough for multiple rooms)

- **Note:** 14 gauge is suitable for longer runs and higher power speakers

4. Speaker/Amplifier - Room's audio system

- Use existing speakers or purchase separately

- Ensure speakers are compatible with your amplifier

Total Cost Per Room: ~$6-8 (relay + wires, excluding speakers and speaker wire which are shared across rooms)

Shared Components (One-Time Purchase):

- **Speaker Wire:** 300 FT roll (~$45.99) - Shared across all rooms

- **Jumper Wires:** 40-piece pack (~$3.99) - Shared across all rooms

## Integration with Existing Features

Room speaker control integrates with:

- ✅ **Amplifier Control** - Works alongside main amplifier

- ✅ **Radio Control** - Independent from radio muting

- ✅ **Light Control** - Lights still flash regardless of speaker mute

- ✅ **Display Configuration** - Works with room and main-station displays

- ✅ **Night Mode** - Respects nighttime muting

## Support

For room speaker issues:

1. Check GPIO pin assignments

2. Verify relay wiring

3. Test relay manually with GPIO commands

4. Check room configuration in backend .env`

5. Verify frontend room settings

6. Check browser console for errors